UNEWS. Leather

Information Bulletin, Nº 20

TECHNICAL AND SALES MEETING

ÍNDEX

## Index

INTRODUCTION

The biannual technical and sales meetings held  at our Application Laboratories are an important tool for technical information update, and involves both products and studies conducted at the lab and experiences and developments reported by our technical and sales representatives. Also, the technical & sales team is briefed on sales poli- cies, and objectives are set.

On the other hand, in-person contact and lively discussion of different points of view adds to the attendees’ general knowledge.

This issue contains structured pieces of work submitted at the meeting, as well as several for- mulas submitted by our commercial  technicians.

Automotive Leather:

- Fogging

- Requirements

- Drying Systems.

Neutralization-Anionization:

Physical Properties

Waterproofing Process with Retanal PSH-200

Influence of Retanning 1 Softness Degree

- Thickness

- Water Absorption

- Water  Vapour Permeability

List of Formulas Submitted

Designed, Edited & Published by Cromogenia’s Leather Department

This bulletin is addressed to all Technicians, Representatives, Customers and Friends who wish to keep themselves updated about our new products and  releases.

For further information or suggestions, please contact: unewsleather@cromogenia.com

(a)

(b)

500μm

(c)

500μm

Fogging

The Fogging test in one of the most important tests per- formed in the automotive leather industry. Fogging results from the heat-induced evaporation of chemicals from the materials used in car interior accessories.

These chemicals are condensed in car windows and form a layer that reduces or even obstructs the driver’s view.

The purpose of this test is to simulate and quantify the eva- poration and condensation of the materials components used in car leather upholstery. This is performed by means of three different methods:

- The Gravimetric Method

- The Reflectometric Method

- The VOC Method

The Gravimetric Method allows determining the amount of evaporated and condensed substances on aluminum foil under specific time and temperature conditions. The result is expressed in mg ―the lower the value, the better the fogging.

The Reflectometric Method allows assessing the extent to which substances evaporated under specific time and temperature conditions induce window fogging. The degree of fogging is measured on a glass plate and the result is ex- pressed in % ―the higher the value, the better the fogging. Tiny, evenly distributed droplets on the glass plate must be obtained.

The VOC Method allows determining volatile organic compounds, mainly including:

- Compounds of carbon with mostly H, O, F, Cl, Br, S, and N.

- Solvents: benzene, toluene, nitrobenzene, xylene, etc.

- Evaporation of organic solvents and tobacco smoke.

The result is expressed in ppm―the lower the value, the better the fogging.

The HAZE Method uses the same principles as the Reflectometric  Method,  but  uses  the  light  that passes

through the glass instead of the light reflected on it. Light is measured with a Hazemeter detector. The result is ex- pressed in % ―the lower the value, the better the fogging.

| STANDARD | TITLE | METHOD |
| --- | --- | --- |
| DIN 75201 A / B | Determination of the fogging characteristics of trim materials in the interior of automobiles. | Reflectometric Gravimetric |
| ISO 17071 IUP 46 | Leather – Physical and Mechanical Tests – Determination of fogging characteristics. | Reflectometric Gravimetric |
| SAE J 1756 | Test method for determination of the fog- ging characteristics of trim materials in the interior of automobiles. | Reflectometric Gravimetric |

## The SAE J1756 method includes several temperature and time  variables.

Examples: Reflectometric Fogging

| STANDARD | SAMPLE DRYING | RESULTS |
| --- | --- | --- |
| DIN 75201 A | 7 days | 50 -65 % |
| SAE J1756 | 7 days | 80-90 % |
| SAE J1756 | Without drying | 80-90 % |

DIN 75201 A: 3h, 100ºC, cooling at 21ºC SAE J1756: 6h, 85ºC, cooling at 38ºC

Reflectometric fogging is quite unreliable because values can be considerably reduced by small variations in leather processing or product composition. The method’s reproducibility is poor and scattered results are obtained. What are the products that affect fogging?

All products used from beamhouse to finishing affect fogging to a greater or lesser extent.

According to Starting  Materials:

| INFLUENCING FACTOR | REPERCUSSION | RECOMMENDATIONS |
| --- | --- | --- |
| Natural fat | Influence on refl. fogging | Select a good degreasing agent |
| Biocidal products | Refl. and grav. fogging | Test / Select |

According to Work  Processes:

| INFLUENCING FACTOR | REPERCUSSION | RECOMMENDATIONS |
| --- | --- | --- |
| Washing | Refl. and grav. fogging | Wash thoroughly after each operation |
| Washing temperature | Refl. and grav. fogging | Wash at high temperatures |
| Drying | Refl. and grav. fogging | Dry at the end with temperature |

According to  Processes Used:

| INFLUENCING FACTOR | REPERCUSSION | RECOMMENDATIONS |
| --- | --- | --- |
| Ammonium salts | Refl. fogging | Do not use / Wash thoroughly |
| Degreasing / tensioactive agents | Refl. and grav. fogging | Test |
| Inorganic tanning agents | Generally without effect | Test |
| Synthetic and vegetal agents | Generally without effect | Test |
| Acrylic agents | Generally without effect | Test |
| Fatliquors | Refl. and grav. fogging | Test |

Current fogging requirements by automobile  manufacturers

In Europe:

Gravimetric fogging (more reliable and more reproducible   results)

In North America:

Reflectometric fogging and, to a lesser extent, gravimetric    fogging.

In Asia:

Haze  and  reflectometric fogging.

Requirements vary greatly according to Country, manufacturer, etc. The fogging values that are valid in/ to most of them are shown in Table 1.

Regarding the steering wheel’s physical resistance,

a high TEAR RESISTANCE value ―greater than 40 N― and an ELONGATION percentage between 15 and 20%, are required. ELONGATION requirements are increased from 20 to 40% in seats.

Requirements

| METHOD | OPTIMAL VALUE | OBJECTIVE | RANGE |
| --- | --- | --- | --- |
| Gravimetric (mgr) | 0 | <3 | >3-5 |
| Reflectometric (%) | 100 | 80 | <60 |
| VOC (ppm) | 0 | 200 | >500 |

Table nº 1 Drying Systems

Different Drying Procedures

Each different drying procedure exerts great influence on the final product, particularly on its physical properties. Three basic drying procedures are used, namely:

- Wet toggle drying:

Sammy-set Wet toggling Stake

Dry milling Dry toggling

- Vacuum

Sammy-set

Vacuum (under temperature)

Stake

Dry milling

Dry toggling (optional)

- TAIC system (The first four

operations are run serially)

Sammy-setr Wet stake TAIC

Dry stake

Milling

Dry toggling (optional) or vacuum

Recommended Products

- Retanning

Retanal A-4 Retanal CAU Retanal RC-200 Retanal RCN-40 Retanal PNB Retanal PR-199/6 Retanal XD Retanal SUL Retanal RST

- Neutralization

Retanal A-4 Neutragent NT

- Fatliquoring

Unix P-48

Fosfol AUT C-3 Extra Fosfol AUT C-7 Extra Fosfol LC-80 Extra

## Sistema TAIC

Neutralization-Anionization

Physical Properties

The  present  study  addresses  the  influence  of

neutralization – anionization on the following leather properties:

- Softness degree

- Physical resistance

An optimal neutralization process involves the following steps:

- Add 2% sodium formate to de-acidify the leather

surface while providing masking of chrome.

- Add 2-3% neutralizing agent sintan and run until uniform pH and anionization values are obtained (B.G. and/or U.I and methylene blue control), both in section and in surface.

As shown by the chemical mechanism, an initially chrome tanned hide previously de-acidified at pH=4.3 with sodium formate has a complex total load of 2+. The neutral salt of naphthalenesulfonic acid (RETANAL A-4) is introduced inside the complex by changing the load to 1-. This is how the

leather is neutralized and anionized.

Influence of  Neutralization on Leather Properties

Chemical Mechanism of Neutralization / Anionization

pH = 4,3

Leather treated with (cationic) sodium formate

+ H2O

pH = 5,0

Leather treated with sodium formate

+ naphthalenesulfonic acidsodium formate

A standard process was used to assess the influence of final pH, and final pH was adjusted to the values of 4.5- 5.5-6.5 for comparison with the same process but without the addition of RETANAL A-4.

NEUTRALI ZATION- ANIONIZATION

## Neutralization-Anionization

PROCESS VARIABLES (LEFT HALVES - REFERENCES)

SOAKING

500% Water at 35º C 0.2% CELESAL DL

and wash 10 min

RETANNING

100% Water at 35º C

4% Chromium salt 33º Sch 2% Sodium formate

Wash 10 min

NEUTRALIZATION

100% Water at 30º C 2% Sodium formate

X% Sodium bicarbonate

Run 2 hours;pH=3.8;  Drain

Run 60 min

Run 60 min; pH=3.7. Drain.

Run 30 min

Run 60 min (pH=adjust) drain

and wash 10 min (control with METHYLENE BLUE)

DYING / FAT-LIQUORING

60% water at 30º C

2% RETANAL HD

3% dying agent 100% water at 60º C

10% FOSFOL AUT C-3

1.5% formic acid

1.5% formic acid

pH = 3.9; Drain and wash 10 min

Run 45 min

Run 45 min

Run 60 min

Run 30 min

Run 30 min

As shown in the reference process (without RETANAL A-4), only neutralization is observed, with no anionization (colorless methylene blue section) and partial penetration of the dying agent.

PROCESS VARIABLES (RIGHZT HALVES)

SOAKING

500% Water at 35º C 0.2% CELESAL DL

and wash 10 min

RETANNING

100% Water at 35º C

4% Chromium salt 33º Sch 2% Sodium formate

Wash 10 min

NEUTRALIZATION

100% Water at 30º C 2% Sodium formate

2% RETANAL A-4

X% Sodium bicarbonate preestablished values)

DYING / FAT-LIQUORING

60% water at 30º C 2% RETANAL HD

3% dying agent 100% water at 60º C

10% FOSFOL AUT C-3

1.5% formic acid

1.5% formic acid

pH = Empty bath and wash 10 min

Run 2 hours;pH=3.8;  Drain

Run 60 min

Run 60 min; pH=3.7. Drain.

Run 30 min

Run 60 min

Run 60 min (Adjust pHs  to

Run 45 min

Run 45 min

Run 60 min

Run 30 min

Run 30 min

In contrast, total anionization is obtained with the addition of RETANAL A-4, as shown with methylene blue, and complete dye penetration is achieved.

## Neutralization-Anionization

VARIATIONS IN LEATHER PROPERTIES

The assessment of SOFTNESS DEGREE and PHYSICAL RESISTANCE suggests that:

  - Increased values (▲) are obtained in the hides treated with RETANAL A-4, at the same pH values.

  - The maximum value is obtained at pH = 5.5.

|  | SOFT | NESS |  |
| --- | --- | --- | --- |

10

9

8

% SOFNESS

7

6

5

4

3

2

1

0

4,5

|  | ELON | ATION |  |
| --- | --- | --- | --- |

20

G

18

% ELONGATION

16

14

12

10

8

6

4

2

0

4,5

5,5

pH

5,5

pH

6,5

6,5

20

|  | TEAR | LOAD |  |
| --- | --- | --- | --- |

18

16

% TEAR LOAD

14

12

10

8

6

4

2

0

4,5

| T | NSILE S | TRENGT | H |
| --- | --- | --- | --- |

20

% TENSILE STRENGTH

18

16

14

E

12

10

8

6

4

2

0

4,5

5,5

pH

5,5

pH

6,5

6,5

These values can be interpreted from the fatliquoring fixation mechanism to the hide. Fatliquoring samples were obtained at different stages of the process, and thin layer chromatography (TLC) was performed. Neutralization conducted at pH=4.5 led to very fast fixation of sulfited fatliquoring and very superficial fatliquoring. Neutralization at 5.5 grants optimal penetration and fixation conditions, as evidenced by TLC with approximately 60-70% fatliquoring exhaustion before the addition of formic acid, and completion with such addition. Neutralization at pH=6.5 leads to fatliquoring fixation only after the addition of formic acid, and also involves superficial fatliquoring.

INFLUENCE OF THE NEUTRALIZATION-ANIONIZATION PH ON FATLIQUORING BATH EXHAUSTION

pH = 4,5

pH = 5,5

pH = 6,5

Antes

Antes

Antes

2min Formic Final	2min Formic Final	2min  Formic Final

Neutralization at pH = 5.5 leads to total, uniform, deep fatliquoring penetration, thus allowing perfect lubrication of the leather and consequently one of the highest physical   resistances.

PSH-200

1 Softness Degree

In order to ascertain the influence of several retanning agents on the water repellency values and the physical properties of several retanning products, a study was designed which consisted

E

in comparing the left half of a hide ―treated with a standard formula― with its right half ―treated similarly but adding 2.5% of retanning active matter (a.m).

Seven wet blue leathers shaved to 2-2.2 mm and with a mean weight of 11 kg/leather were used  as raw material.

The retanning agents selected belong to several types of our range and also include a vegetal agent (acid chestnut) widely used in water repellency processes.

#### RETANAL SUL ACID CHESTNUT

RETANAL MV EXTRA RETANAL CLE RETANAL RCN-40 RETANAL RC-200 RETANAL RST

The standard process was conducted as  follows:

On W.B. weight

WASHING

200% water at 35ºC

0.4% formic acid	Run 20 min  Drain

RETANNING

100% water at 35ºC

4% Chromium salt 33º Sch	Run 45 min 2% Sodium formate Run 30 min	pH=3.8

0.2% Sodium bicarbonate	Run 60 min   pH=4.0	Drain

WASHING

200% water at 35ºC	Run 10 min  Drain NEUTRALIZATION

100% water at 35ºC

2% Sodium formate	Run 15 min

3% RETANAL A-4	Run º 90 min pH=4.8

0.2% Sodium bicarbonate	Run 120min  pH=5.2	Overnight bath

WASHING

200% water at 35ºC	Run 15 min  Drain

DYE / FAT-LIQUORING

50% water at 35ºC

2.5% a.m. RETANNING/ REFERENCE Run 15 min

6% REPELAN PSH-200	Run 10 min

1.5% RETANAL A-4

3.5% Acid Black 210	Run 30 min Penetration control 100% Water at 60ºC

0.7% Formic acid	Run 20 min

1% Acid Black 210	Run 20 min

  - % Formic acid	Run 20 min  pH=4. Drain

TOP

100% Water at 60ºC

4% REPELAN PSH-200	Run 10 min

  - % Formic acid	Run 20 min  pH=3.8. Drain

WASHING

200% Water at 35ºC	Run 15 min Drain FIXATION 150% Water at 35ºC

4%    Chromium salt 33º Sch	Run 60 min  pH=3.1.. Drain

WASHING

200% Water at 30ºC	Run 10 min  Drain WASHING

200% Water at 30ºC	Run 10 min  Drain

DRAIN-VACUUM 3 min at 60ºC - AIR-DRY - STAKE

All data assessed until the submission of this study are shown in three graphs for each retanning agent.

All measurements exceeded 150.000 MAESER, and therefore the water repellency values shown in the graphs correspond to water absorption at 15.000 Maeser and to water vapor permeability.

4 Water Vapour PermeabilitPy

SH-200

2 Thickness, 3 Water Absorption,

RETANAL SUL

| SOFTNESS THICKNESS FIRMNESS 14 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 8 |  |  |  |  |
|  |  | 6,4 |  |  |  |  |  | HEAD |
|  |  |  | 1,3 |  |  |  |  | BUTT |
|  |  |  |  |  | -2 | -1,5 BELLY |  |  |
| -4,9 |  |  |  |  |  |  |  |  |

20

15

INCREASE (%)

10

5

0

-5

-10

REFERENCE

6,8   6,4

SUL

-6

VARIATION

WATER VAPOR PERMEABILITY

(mg/cm2*h)

|  |  | 16 |  |
| --- | --- | --- | --- |
|  | 7,02 8,13 |  |  |
|  |  |  | REFERENCE |
| SUL |  |  |  |
| VARIATION |  |  |  |

20	20

ABSORTION (%)

15	15

10	10

5	5

0	0

-5	-5

-10

20

-10

Increased head thickness and firmness (8% and 14%, respectively). While no significant changes are  observed in  the  absorption %,  water vapor permeability is  increased by  16%.

## ACID CHESTNUT

SOFTNESS

THICKNESS

FIRMNESS

14,2

HEAD

1,5

0,7

BUTT

9

5,3

1

BELLY

-7,3

15

INCREASE (%)

10

5

0

-5

-10

15

ABSORTION (%)

10

5

0

-5

-10

-15

12

REFERENCE

ACID CHESTNUT

VARIATION

9,15

9,7

6

WATER VAPOR PERMEABILITY

(mg/cm2*h)

|  | 6,5 5,5 |  |  |
| --- | --- | --- | --- |
|  |  |  | REFERENCE |
|  |  |  | ACID CHESTNUT |
|  |  |  | VARIATION |

11

10

9

8

7

6

5

4

3

2

1

-15	0

Increased head thickness and firmness (9% and 14.2%, respectively). 15% decrease of water absorption. No other significant values obtained.

PSH18

-2SOFT0NESS

0THICKNESS

FIRMNESS

BUTT

RETANAL MV EXTRA

20

16

INCREASE (%)

14

12

10

8

6

4

2

0

- 2

- 4

|  |  |  | REFERENCE |  |
| --- | --- | --- | --- | --- |
|  |  |  |  | MV EXTRA |
|  |  |  |  | VARIATION |

5	5    4,9

REFERENCE

MV EXTRA

VARIATION

9,15

9,7

6

WATER VAPOR PERMEABILITY

(mg/cm2*h)

4	12

11

ABSORTION (%)

3	10

9

2	8

7

1	6

5

0	4

3

-1	2

1

- 2	-2	0

Notably increased firmness and thickness.

## RETANAL CLE

SOFTNESS

THICKNESS

FIRMNESS

HEAD

BUTT

BELLY

X

HEAD BELLY

X

INCREASE (%)

X

X

X

0

|  | 5,5 5,4 |  |  |  |
| --- | --- | --- | --- | --- |
|  |  |  | REFERENCE |  |
|  |  |  |  | CLE |
|  |  |  |  | VARIATION |

X 6

5

CLE

VARIATION

11,83

10,06

10

REFERENCE

ABSORTION (%)

WATER VAPOR PERMEABILITY

(mg/cm2*h)

4	12

3	11,5

2	11

1	10,5

0	10

E

- 1

- 2	- 2

Increased firmness and thickness. Increased water vapor permeability (10%).

9,5

9

RETANAL RCN40	PSH-200

|  |  |  |  |  |  |  |  | HEAD |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | BUTT |
|  |  |  |  |  |  |  |  | BELLY |  |  |
| SOFTNESS THICKNESS FIRMNESS |  |  |  |  |  |  |  |  |  |  |

X

INCREASE (%)

X

X X X X X X 0

- X

REFERENCE

RCN40

VARIATION

6,4

5,9

X 8

|  |  | 11,83 |  | REFERENCE |
| --- | --- | --- | --- | --- |
|  | 8,76 |  |  | RCN40 |
|  |  |  |  | VARIATION |

6	35	35

WATER VAPOR PERMEABILITY

(mg/cm2*h)

4	30

ABSORTION (%)

2	25

0	20

- 2	15

- 4	10

- 6	5

- 8	- 8	0

35

INCREASE (%)

30

25

20

15

10

5

0

- 5

- 10

- 20

4,5

Notably increased firmness (40%) and thickness, with no significant variations in softness. Decreased water absorption (8%) and strongly increased permeability  (35%).

SOFTNESS

THICKNESS

30

HEAD

BUTT

BELLY

13

4,7

6,7

-11,4 -10,5 -11,3

FIRMNESS

RETANAL RC-200

|  | 4,3 | 4,3 |  |
| --- | --- | --- | --- |
|  |  |  | REFERENCE |
|  |  |  | RC-200 |
|  |  |  | VARIATION |

4

3,5

11

8,5

7,63

REFERENCE

RC-200 VARIATION

WATER VAPOR PERMEABILITY

(mg/cm2*h)

3	12

ABSORTION (%)

2,5	10

2	8

1,5	6

1	4

0,5	2

0	0	0

Evenly decreased softness in the different parts of the hide. Significantly increased thickness and strongly increased firmness (30%). No variations in water absorption, and increased water vapor permeability (11%).

E

PSH-200

## RETANAL RST

SOFTNESS

THICKNESS

FIRMNESS

XX

HEAD

XX   XX

BUTT

BELLY

XX

XX

XX

XX

X

X X

INCREASE (%)

X

X X

0

X X X

15

8,18  8,3

REFERENCE

RST VARIATION

|  | 4,5 4,3 |  |  |
| --- | --- | --- | --- |
|  |  |  | REFERENCE |
|  |  |  | RST |
|  |  |  | VARIATION |

5	20

4	18

WATER VAPOR PERMEABILITY

(mg/cm2*h)

3	16

ABSORTION (%)

2	14

1	12

0	10

-1	8

- 2	4

- 3	2

- 4	- 4	0

Decreased degree of softness in head (9.8%) and significantly increased thickness in head, butt and belly; thickness is increased in the entire leather ―more in head and less in butt and belly. Strongly increased firmness (25%).

In respect of water repellency value, slightly decreased water absorption (4%), and increased water vapor permeability (15%).

# LIST OF FORMULAS

SUBMITTED

## List of Formulas Submitted

### Sheepskin

Goat Suede-1 Goat Suede-2

Goat Vegetable pickle-1 Goat Vegetable tanned-2

Goat Vegetable dye/fat-liquoring-3 Goat Vegetable-4

Goat crust for nubuck Goat and cow shrunken

Goat black vegetable nubuck Lamb dry White

Lamb Nappa dry White footwear Lamb Nappa dry White garment Lamb Nappa finishing garment Lamb Nappa scoured footwear White Double Face

Cow

Soaking-unhairing for wet white Deliming-bating-tanning for wet  white

Automotive leather w.b. 1,2 m.m. (AV-4512-3) Automotive leather w.b. 1,2 m.m. (AV-6512) Upholstery 1,2 m.m.  (AV-6412)

Upholstery 1,2 m.m.  (AV-6412-2)

High-Resistance Urethane Tops

C0002

C./ 40, Nº  14-16

08040 BARCELONA SPAIN

Tel. +34 93 432 94 00

Fax +34 93 422 60 14

cromogenia@cromogenia.com www.cromogenia.com